EXPERIMENT 6

| Name: Arav Desai | Roll No: 3 |
| --- | --- |
| Subject: DE | Class/Batch: AIML A/ B-1 |
| Date of Performance: | Date of Submission: |

Theory

Multiplexer (MUX)

A multiplexer is a combinational logic circuit that selects one input line from several input signals and forwards it to a single output line. It's often referred to as a "data selector." Multiplexers are widely used in communication systems, data routing, and signal processing.

Working Principle:

- Inputs and Selection Lines: A multiplexer has multiple data inputs, a single output, and selection lines. The number of selection lines (n) determines how many input lines (2ⁿ) the multiplexer can handle.

- Selection Process: The state of the selection lines determines which input is connected to the output. For example, in a 4-to-1 MUX, there are 4 inputs, 2 selection lines, and 1 output.

Truth Table Example (for 4-to-1 MUX):

| S0 | S1 | Y |
| --- | --- | --- |
| 0 | 0 | I0 |
| 0 | 1 | I1 |
| 1 | 0 | I2 |
| 1 | 1 | I3 |

Here, I0, I1, I2, and I3 are the input signals, and S1 and S0 are selection lines.

Applications:

- Data transmission in communication systems.

- Switching circuits in telecommunication networks.

- Display systems.

De-Multiplexer (DEMUX)

A de-multiplexer, or DEMUX, is essentially the opposite of a multiplexer. It takes a single input signal and distributes it to one of the many output lines based on the selection line values. It's often referred to as a "data distributor."

Working Principle:

- Inputs and Outputs: A de-multiplexer has one data input, multiple output lines, and selection lines.

- Distribution Process: The state of the selection lines determines which output line will receive the input signal. For example, in a 1-to-4 DEMUX, there is 1 input, 2 selection lines, and 4 outputs.

Truth Table Example (for 1-to-4 DEMUX):

| S0 | S1 | Y |
| --- | --- | --- |
| 0 | 0 | O0 |
| 0 | 1 | O1 |
| 1 | 0 | O2 |
| 1 | 1 | O3 |

Here, O0, O1, O2, and O3 are the output lines, and S1 and S0 are selection lines.

Applications:

- Data distribution in communication systems.

- Decoding information in computer systems.

- Signal routing in electronic systems.

Key Difference Between MUX and DEMUX:

- Multiplexer: Multiple inputs, single output.

- De-Multiplexer: Single input, multiple outputs.

- MUX performs data selection, while DEMUX performs data distribution.

Circuit

Multiplexer Circuit:

De-Multiplexer Circuit:

Conclusion

In conclusion, multiplexers and de-multiplexers are essential devices in digital electronics and communication systems, having opposite functionalities, to facilitate effective data transmission management. Demultiplexers distribute a single input signal among multiple outputs based on selection criteria, in contrast to multiplexers, which select and forward the desired input signals to a single output.

Assessment

| Timely Submission (7) | Presentation (06) | Understanding (12) | Total (25) | Sign |
| --- | --- | --- | --- | --- |